# V3_two_sensor_IMU_code.py — Technical Documentation

This documents exactly what V3_two_sensor_IMU_code.py does as it currently stands, including the sensor-stability indicator added on top of the original script. For physical wiring, see BNO055_Wiring_Guide.md. For a browser-based view of this same data, see imu_web_dashboard.py (covered at the end of this document).

## 1. What this script does

Reads orientation from one or two Bosch BNO055 9-DOF IMU sensors on a shared I2C bus, and for each sensor prints:

- Roll, Pitch, Yaw in human-readable degrees

- Calibration status (System / Gyro / Accel / Mag, each 0–3)

- Stability status — whether the sensor's output is currently steady or fluctuating (see Section 5)

Every successful reading is also appended to a CSV log file for later analysis.

## 2. Requirements

Hardware

- Raspberry Pi (any model with an I2C header — this was built/tested against a Raspberry Pi 5)

- One or two BNO055 breakout boards

- Jumper wires

Software

sudo pip3 install --break-system-packages \

adafruit-circuitpython-bno055 \

adafruit-blinka

I2C must be enabled first: sudo raspi-config → Interface Options → I2C → Enable.

Confirm both sensors are visible before running the script:

i2cdetect -y 1

# Expect entries at 0x28 and 0x29

## 3. Configuration reference

All of these are constants near the top of the file — edit them directly to change behavior, no command-line flags needed.

| Constant | Default | What it controls |
| --- | --- | --- |
| SENSOR_1_ADDRESS | 0x28 | I2C address of the first sensor (ADR pin → GND) |
| SENSOR_2_ADDRESS | 0x29 | I2C address of the second sensor (ADR pin → 3.3V) |
| SINGLE_SENSOR_MODE | False | Set True to run with only sensor #1 connected |
| READ_INTERVAL_S | 0.1 | Poll rate — 0.1 = 10Hz |
| READ_RETRIES | 2 | Retries on a transient I2C read error before giving up that cycle |
| RETRY_DELAY_S | 0.01 | Delay between retries |
| CALIBRATION_READ_EVERY_N_CYCLES | 10 | How often calibration status is re-read (it changes far slower than orientation, so it isn't read every cycle) |
| I2C_FREQUENCY_HZ | 400000 | I2C bus speed (Fast Mode). Helps only marginally on the BNO055 specifically — see the comment in the script for why |
| PRINT_EVERY_N_CYCLES | 1 | How often a reading is printed to the console. Raise this (e.g. 5) if console output is too fast to read |
| CSV_FLUSH_EVERY_N_CYCLES | 1 | How often the CSV file is flushed to disk |
| CSV_LOG_DIR | "logs" | Folder the CSV log is written into (relative to wherever you run the script from) |
| STABILITY_THRESHOLD_DEG | 2.0 | Added. Max allowed change (degrees) in Roll/Pitch/Yaw between two consecutive readings before a sensor is flagged UNSTABLE — see Section 5 |

## 4. How the read loop works

- validate_config() — checks that none of the cycle-throttling constants above are set to 0 (which would crash the loop with a divide-by-zero); exits cleanly with a clear message if so.

- setup_sensors() — opens the shared I2C bus and connects to whichever sensor(s) are configured. If neither sensor connects, the script exits with a message pointing at i2cdetect -y 1.

- run_read_loop() — the main loop:

  - Reads each sensor (BNO055Sensor.read_orientation()), which retries transient I2C errors up to READ_RETRIES times before giving up for that cycle and returning None.

  - Computes each sensor's stability status (see Section 5).

  - Prints the reading + stability line (throttled by PRINT_EVERY_N_CYCLES).

  - Logs the reading to CSV (a failed/None reading is silently skipped — not written as a row).

  - Holds a fixed sample rate using time.monotonic(), so a slow cycle (e.g. one that hit a retry) doesn't cause the whole run to drift behind schedule.

- On Ctrl+C, main() flushes and closes the CSV file and releases the I2C bus (i2c.deinit()) before exiting — skipping that release step is a known cause of the next run failing to reopen the I2C bus without a reboot.

## 5. Sensor stability indicator

This is the addition made on top of the original script. It answers a different question than the calibration numbers do:

- Calibration status (Cal S:_ G:_ A:_ M:_) reports the sensor's own internal calibration state.

- Stability status (new) reports whether the actual values coming out are currently jumping around or holding steady — a more direct answer to "is this sensor behaving reliably right now?"

How it works (get_stability_status()): each cycle, the current Roll/Pitch/Yaw is compared to that same sensor's previous reading.

- If any of Roll, Pitch, or Yaw changed by more than STABILITY_THRESHOLD_DEG (default 2.0°) since the last reading, the sensor is reported UNSTABLE (fluctuating).

- Otherwise it's STABLE.

- Three edge cases are handled explicitly:

  - No data this cycle → N/A (no data)

  - No previous reading yet (first cycle for that sensor) → N/A (first reading)

  - Yaw wraparound: Yaw is a 0–360° compass heading, so a change from 359° to 1° is really only a 2° rotation, not a 358° jump. The comparison corrects for this so a sensor sitting still near that boundary isn't falsely flagged as unstable.

What you'll see in the console:

[IMU-1 @ 0x28]  Roll:   -3.75°   Pitch:  177.56°   Yaw:  274.31°   (Cal S:3 G:3 A:0 M:3)

-> IMU-1 status: STABLE

[IMU-2 @ 0x29]  Roll:    2.10°   Pitch:   45.03°   Yaw:  118.92°   (Cal S:0 G:1 A:0 M:0)

-> IMU-2 status: UNSTABLE (fluctuating)

Common cause of UNSTABLE, especially on Yaw specifically: the magnetometer hasn't calibrated yet (Cal ... M:0). Rotating the sensor through a figure-8 motion, then holding it briefly in a few different tilted orientations, is what raises magnetometer calibration — see BNO055_Wiring_Guide.md for the full calibration procedure and troubleshooting table.

Design notes:

- This is purely additive — nothing in BNO055Sensor or the original read/print/log logic was modified to add it.

- Stability is tracked per-sensor-name in a module-level dictionary (_last_readings_for_stability), independent of the calibration tracking already inside BNO055Sensor.

- The stability check runs every cycle (not just when printing), so the "previous reading" used for comparison always reflects the true previous cycle rather than the previous printed one.

## 6. CSV log format

Written to logs/imu_log_<timestamp>.csv by default (a fresh file per run). Columns:

| Column | Meaning |
| --- | --- |
| timestamp | ISO 8601 timestamp (millisecond precision) of this reading |
| sensor_name | IMU-1 or IMU-2 |
| address | I2C address as hex string (e.g. 0x28) |
| roll_deg, pitch_deg, yaw_deg | Orientation in degrees |
| cal_system, cal_gyro, cal_accel, cal_mag | Calibration status, 0–3 each |

Note: the stability status is not currently written to this CSV — it's console-only. (The companion web dashboard, below, computes its own stability independently from this same CSV, using the last two logged rows for each sensor.)

## 7. Companion: web dashboard

imu_web_dashboard.py is a separate script that displays this same data in a browser instead of the terminal — useful since SSH terminals can be hard to read live, especially over a laggy connection.

- It only reads the CSV file this script writes — it never imports or talks to V3_two_sensor_IMU_code.py directly, so running, stopping, or crashing the dashboard has zero effect on the sensor script.

- It shows the same Roll/Pitch/Yaw/calibration data, plus its own independently-computed STABLE / UNSTABLE badge and a live/STALE indicator (stale = no new CSV row for that sensor in the last few seconds, meaning the main script likely stopped or that sensor disconnected).

To run both together:

# Terminal 1 — unchanged:

python3 V3_two_sensor_IMU_code.py

# Terminal 2:

python3 imu_web_dashboard.py

Then open http://<pi-ip-address>:5000 from a phone or laptop browser on the same network. Find the Pi's IP with hostname -I.

## 8. Known limitations

- Stability and calibration status are not the same signal and can disagree briefly — e.g. right after picking the sensor up to calibrate it, it will correctly show UNSTABLE even as calibration numbers are climbing, since it's genuinely moving at that moment.

- The stability check has no memory beyond one previous reading — it can't distinguish "briefly unstable due to one noisy sample" from "genuinely and persistently unstable." For that, watch the indicator over several consecutive cycles rather than a single one.

- STABILITY_THRESHOLD_DEG is a single fixed value for all three axes (Roll, Pitch, Yaw). Yaw is inherently noisier pre-calibration than Roll/Pitch, so it's the axis most likely to trip UNSTABLE first — this is expected, not a bug.