# BNO055 IMU Hardware Safety & Wiring Reference Manual

This document provides hardware wiring specs, electrical operating limits, physical handling procedures, and safety protocols for deploying single and dual BNO055 9-DOF Orientation Sensors across microcontrollers and single-board computers.

## 1. Electrical & Thermal Operating Limits

Operating the BNO055 outside these absolute maximum ratings can cause permanent hardware degradation, I2C bus latch-ups, or premature sensor failure.

| Parameter | Minimum | Nominal | Maximum | Notes / Units |
| --- | --- | --- | --- | --- |
| Supply Voltage (VIN) | 3.3 V | 3.3V – 5.0V | 5.5 V | Dependent on breakout regulator |
| Logic I/O Voltage (SDA, SCL, ADR) | -0.3 V | 3.3 V | 3.6 V | Absolute max for raw chip pins |
| Operating Current (IDD) | 12.3 mA | 12.5 mA | 30.0 mA | During full fusion mode |
| Operating Temperature | -40 °C | +25 °C | +85 °C | Sensor fusion degradation above 70°C |
| I2C Clock Frequency | 10 kHz | 100 kHz | 400 kHz | Clock stretching support required |

## 2. Hardware Wiring Matrix & Pinouts

To operate two BNO055 sensors on a single I2C bus without address collisions:

- Sensor 1 (0x28): Connect ADR to GND (0V logic).

- Sensor 2 (0x29): Connect ADR to 3.3V (HIGH logic).

### System Connection Table

| BNO055 Pin | Pin Function | Sensor 1 (0x28) | Sensor 2 (0x29) | Arduino Uno / Nano | Arduino Mega | ESP32 | Raspberry Pi (Physical Pin) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VIN | Main Power Input | Power Source | Power Source | 5V Pin | 5V Pin | 3.3V Pin | Pin 1 (3.3V Power) |
| GND | System Ground | Common GND | Common GND | GND Pin | GND Pin | GND Pin | Pin 6 (Ground) |
| SDA | I2C Serial Data | Shared SDA | Shared SDA | A4 | Pin 20 | GPIO 21 | Pin 3 (GPIO 2 / SDA1) |
| SCL | I2C Serial Clock | Shared SCL | Shared SCL | A5 | Pin 21 | GPIO 22 | Pin 5 (GPIO 3 / SCL1) |
| ADR / ADD | Address Select | GND | 3.3V / VCC | GND | GND | GND / 3.3V | Pin 9 (GND) / Pin 17 (3.3V) |
| RST | Hardware Reset | Optional | Optional | Digital Pin | Digital Pin | GPIO Pin | GPIO Pin (NC if unused) |
| INT | Hardware Interrupt | Optional | Optional | Interrupt Pin | Interrupt Pin | GPIO Pin | GPIO Pin (NC if unused) |

## 3. Essential Electrical & Physical Safety Protocols

### A. Voltage Level Matching & Logic Protection

- 3.3V Native Logic Hosts (ESP32, Raspberry Pi):

  - GPIO lines on Raspberry Pi and ESP32 are not 5V tolerant. Always power the BNO055 from a 3.3V rail.

  - Never feed 5V into the SDA, SCL, or ADR pins on 3.3V microcontrollers/SBCs.

- 5V Logic Hosts (Arduino Uno, Mega):

  - Ensure your BNO055 module is an Adafruit or similar breakout board equipped with onboard 3.3V regulators and bi-directional level shifters.

  - If using raw bare BNO055 ICs, external MOSFET logic level converters are mandatory on SDA and SCL.

### B. Hot-Plugging & Power Order Hazards

- Do Not Hot-Plug: Never connect or disconnect the sensor while the microcontroller or power supply is energized. Voltage spikes or transient ground loops during hot-plugging can instantly blow out internal CMOS gates.

- Common Ground First: Always ensure a solid, shared GND connection between the host board, sensors, and any external power supply prior to applying power to VIN.

### C. ESD (Electrostatic Discharge) Handling

- The BNO055 contains highly sensitive MEMS structures and CMOS electronics with ESD tolerance rated up to 2 kV (HBM).

- Always handle breakout boards by the edges, avoiding direct finger contact with bare chip surfaces, headers, or exposed solder pads.

- Use anti-static wrist straps or ground yourself on a metal surface prior to handling the hardware in low-humidity environments.

## 4. Signal Integrity & Environment Hardening

### A. Cable Length & I2C Bus Pull-Ups

- Pull-Up Resistors: Adafruit BNO055 breakout boards feature onboard 10 kΩ pull-up resistors on SDA and SCL.

  - Paralleling two breakout boards on the same bus yields an effective resistance of 5 kΩ (10 kΩ || 10 kΩ), which is optimal for short wires (< 20 cm).

- Long Cabling (Robotic Arms / ROV Tether Cables):

  - Wire lengths exceeding 30 cm introduce high bus capacitance, leading to corrupt sensor data or I2C Remote I/O Error.

  - Use twisted-pair cabling (twist SDA with GND, and SCL with GND).

  - Add external 2.2 kΩ pull-up resistors to 3.3V near the host controller if wire length exceeds 50 cm.

### B. Magnetic Interference & Placement Safety

- Keep Away from Motors & High Currents: The BNO055 uses a sensitive triaxial geomagnetic sensor for compass heading fusion.

- Minimum Clearances:

  - Maintain at least 10 cm to 15 cm distance from DC motors, stepper motors, solenoids, thrusters, high-current power distribution boards, and battery packs.

  - Avoid mounting the sensor directly onto ferrous metal frames (iron, steel). Use non-magnetic standoffs (nylon, brass, or 3D-printed PLA/PETG).

### C. Mechanical Shock & Vibration Isolation

- MEMS Gyroscope Protection: Heavy mechanical vibration (e.g., from drone propellers or underwater thrusters) creates high-frequency noise that saturates the sensor fusion algorithms.

- Use silicone anti-vibration damping mounts or rubber grommets between the sensor PCB and the robot frame.

- Do not apply excessive mechanical torque when screwing down the PCB mounting holes to prevent warping or micro-fracturing the chip package.

## 5. Pre-Power Safety Checklist

- VIN power supply voltage verified with a multimeter before plugging into the board.

- Raspberry Pi/ESP32 power connected to 3.3V, NOT 5V.

- Common ground (GND) firmly established across all sensors, hosts, and power supplies.

- Sensor 1 ADR tied to GND (0x28).

- Sensor 2 ADR tied to 3.3V (0x29).

- No exposed bare copper wires or uninsulated header pins touching metal chassis/frame.

- Sensors mounted at least 10 cm away from high-current cables, motors, and magnets.